# Disentangling Representation Effects and Normal Reference Matching in Few-Shot Industrial Anomaly Localization

# Abstract

Combining pretrained visual encoders can improve few-shot industrial anomaly localization, but an apparent fusion gain may reflect changed weights, additional information, or altered normal-reference selection. We introduce a controlled decomposition that distinguishes these effects using frozen encoders and identical normal supports. An exact duplicate control isolates reweighting; complementary representation replacements measure the value of additional descriptors at fixed slot or branch-group weights. Crossing these constructions with joint and independent reference matching yields a paired interaction that separates absolute representation effects from their dependence on matching. For DINOv2-S, the two primary interactions on the Metal Parts Defect Detection (MPDD) dataset are +0.772 and +0.595 pixel average-precision percentage points, whereas their directions remain unresolved on the BeanTech Anomaly Detection (BTAD) dataset. Further validation supports positive interactions on MVTec AD and VisA, with VisA treated as in-domain. A separately specified KolektorSDD2 confirmation meets all four directional criteria for DINOv2-S and WideResNet50-2. Encoder and support-set analyses reveal meaningful variation across conditions. The resulting evidence shows why representation composition and reference selection should be assessed together and provides a practical way to distinguish additional-feature value from reweighting and matching effects.

Keywords: few-shot anomaly localization; frozen visual encoders; normal reference matching; fixed feature fusion; representation interaction.

# 1 Introduction

Industrial visual inspection must locate defects whose appearance is difficult to enumerate before deployment. A new product may provide only a few verified normal images, while representative scratches, contamination, missing parts, or structural defects remain unavailable. Industrial anomaly benchmarks reflect this setting through normal training images and heterogeneous test anomalies [1, 2, 3, 4]. Few-shot anomaly localization therefore relies on a small normal support set to identify departures from expected appearance without training a target defect classifier.

Frozen pretrained encoders provide local descriptors that can be compared directly with a memory of normal support patches. AnomalyDINO demonstrates the effectiveness of DINOv2 features for this purpose [5, 6]. Combining encoders can enrich these descriptors, and recent systems already integrate visual and vision-language pretrained representations [7, 8]. However, comparing the final scores of a two-branch and a three-branch system leaves a central design question unresolved: what makes the additional representation useful?

Two confounding effects complicate that comparison. First, adding an equally weighted branch also changes the weights of existing information. Copying one of two original branches adds no information, yet increases its effective weight from one half to two thirds. Replacing that copy with a different encoder is a distinct intervention whose benefit cannot be read directly from the original two-versus-three comparison. Second, the selected normal reference depends on how branch evidence is combined. Joint matching scores one shared support patch across branches before selecting the best candidate; independent matching allows each branch to select its own closest reference. A new descriptor can alter both the distance evidence and the chosen reference under a fixed matching rule.

This coupling makes absolute representation gains and matching dependence different questions. Independent matching has a lower or equal raw minimum-distance score, but average precision depends on the ranking of normal and anomalous pixels rather than score magnitude alone. Similarly, an added representation may help under both matching rules while showing little evidence that its gain depends on the rule. A controlled comparison must distinguish these possibilities instead of attributing the complete fusion gain to representation diversity.

Prior research establishes the importance of fusion design and consistent neighborhoods [9, 10, 11, 12]. We build on those ideas to isolate a specific unresolved attribution problem in dense industrial localization. With frozen encoders, identical supports and fixed distance weights, we cross two representation replacements with two reference-selection rules. The resulting paired contrasts quantify the value of the added descriptor and the change in that value when the shared-reference constraint is removed. This provides an interpretable basis for deciding whether a fusion improvement comes from reweighting, additional information, or their interaction with reference selection.

The contributions are as follows.

1. A controlled decomposition of additional-representation value. We construct an exact duplicate control that exposes the weight change hidden in an ordinary two-versus-three-branch comparison. Replacing the duplicate at fixed slot weights isolates the representation change, while a complementary construction preserves the combined weight of the original visual branch and the added branch while retaining the AnomalyCLIP branch weight. Together, these controls distinguish new descriptor information from redistribution of existing evidence and make the attribution of a fusion gain testable.

2. A paired characterization of representation–matching dependence. We cross both representation constructions with joint and independent normal-reference matching on the same support bank. The resulting difference-in-differences quantifies how reference selection changes the effect of an added representation. Reporting this interaction alongside absolute representation effects separates two practically different outcomes: a useful representation with little established matching dependence, and a favorable matching interaction without a clear absolute gain. The contribution lies in this controlled formulation of the industrial fusion problem, using established matching operators and statistical contrasts.

3. Evidence that representation–matching interactions vary by encoder and dataset. The primary experiments separate the two outcomes above on MPDD and BTAD. Validation on MVTec AD and VisA, a separately specified KolektorSDD2 confirmation, and matched encoder substitutions extend the evidence while revealing its limits. DINOv2-S has an unresolved interaction direction on BTAD, whereas WideResNet50-2 has positive interactions on both primary datasets; ConvNeXt provides an inconclusive case on MPDD. These findings establish a bounded design insight: representation composition and normal-reference selection need to be assessed jointly. Support-set and correspondence analyses further identify conditions that warrant sensitivity checks.

The study centers on this attribution problem. Its dual-encoder anchor, A1, provides a fixed reference for controlled comparisons; the results support conditional design choices rather than a universal encoder ranking.

# 2 Related Work

Three lines of research inform the attribution of gains in a frozen multi-encoder system: methods for modeling normal appearance and retaining reference features, foundation-model fusion, and neighborhood consistency across representations. We review their capabilities and identify the specific comparison that remains necessary in the present setting.

## 2.1 Normal Appearance Models and Feature Memories

Learned normal models identify anomalies through deviations from expected appearance or feature structure. SLSG combines generative pretraining, simulated anomalies and graph-based embedding modeling [13]. RealNet uses diffusion-based anomaly synthesis with feature and residual selection [14]. PaDiM instead fits multivariate Gaussian distributions to pretrained patch embeddings [15]. These methods offer different ways to estimate normality, with corresponding training, adaptation and sample-size requirements.

Memory-based methods retain observed normal descriptors for local comparison. PatchCore combines a representative feature memory with nearest-neighbor scoring [16], while AnomalyDINO demonstrates effective few-shot localization with frozen DINOv2 patches [5]. Their performance depends on the descriptor, the coverage of the reference memory and the preprocessing that defines the evaluated image region.

Recent work expands these choices beyond adding encoders. FEAD enriches sparse normal patterns through conditional feature transformation and multi-frequency modeling [17]. K-NG studies few-shot online detection using an evolving Neural Gas representation [18]. PGAD combines global invariance, multiscale local evidence and score calibration [19], and FastRef refines normal prototypes at test time [20]. SubspaceAD models frozen descriptors through a principal subspace [21], while DCP-SFR preserves shallow defect information in deep representations [22]. These approaches highlight the roles of reference coverage, cue retention and adaptation. We hold the support bank fixed so that representation replacement and reference selection can be compared without query-conditioned updates.

## 2.2 Foundation Models and Visual Representation Fusion

Contrastive Language–Image Pretraining (CLIP) provides transferable visual and textual representations [23]. WinCLIP combines prompt ensembles and local visual features for zero- and few-shot detection [24]. AnomalyCLIP learns object-agnostic normality and abnormality prompts [8], PromptAD learns from normal target samples [25], and InCTRL uses in-context residual learning with sample prompts [26]. AA-CLIP introduces anomaly-aware alignment [27], FAPrompt develops fine-grained abnormality prompts [28], and UniVAD extends training-free reference-based detection across domains [29]. Their supervision, adaptation and inference paths differ. The AnomalyCLIP visual branch, denoted C, uses only visual descriptors; text scores and learned prompt outputs do not enter anomaly scoring.

Sea-CLIP is especially relevant because it combines CLIP and DINOv2 representations for few-shot detection and includes shared-reference-like and independent nearest-neighbor evidence [7]. M3DM integrates RGB and point-cloud features with multiple memories [30], and CIF uses hypergraph structural commonality to guide multimodal reference memories [31]. The architectural analysis in 3D-ADNAS examines conditional benefits of fusion modules and fusion stages [9]. These studies establish encoder combination and fusion design as existing approaches.

Our emphasis is the attribution of the added representation's effect under controlled weights and references. We replace one descriptor while retaining support identities and slot weights, then measure how its effect changes under a different reference-sharing constraint. This differs from attributing a performance difference between complete architectures to one added representation, because other architectural and adaptation choices remain fixed.

## 2.3 Neighborhood Consistency across Representations

Multi-view anomaly detection studies how neighborhood agreement relates to anomalous behavior. MUVAD uses neighborhoods to distinguish cross-view inconsistency from instances that are unusual across views [11]. Neighborhood Consensus Networks align neighborhood structures [12], ECMOD combines contrastive learning with neighborhood-consistency information [32], and SCoNE addresses consistent local neighborhoods across views [10]. Shared and independent neighborhoods are therefore established concepts with relevance beyond industrial images.

Dense industrial localization introduces a specific setting: verified normal image supports, frozen patch descriptors and pixel-level evaluation. We investigate how changing the branch representation affects performance within this setting, using duplicate and balanced controls to identify the interaction with reference selection. The contribution concerns the controlled representation comparison and its empirical consequences, while the underlying neighborhood concepts come from prior work.

Together, these three strands explain how normal evidence is stored, how its representation is expanded and how references are shared. Existing methods motivate all three choices; our study connects them through a factorized comparison that separates representation gain from reweighting and quantifies its dependence on matching. This distinction allows an observed fusion improvement to be interpreted at the level of the design choice that produced it.

# 3 Framework and Methods

## 3.1 Problem Statement

For category c, the input consists of K verified normal support images and a query image x. The support set is

𝒳c = {xic : i = 1, …, K}	(1)

Here xic is support image i of category c, and the query may be normal or anomalous. Category labels specify the relevant support bank; no target defect class is predicted. We use p for a query patch, r for an aligned support-patch index that contains both support-image identity and spatial position, and b for a visual branch. The same index r denotes the corresponding support location in every branch after spatial alignment. It does not require a query patch to match the same spatial coordinate in its support image.

The outputs are a continuous pixel anomaly map At and an image-level score simg,t, where the matching-rule label t is J or L. A displayed defect contour is derived from the map using a visualization threshold, as specified in Section 3.6. Test labels and masks are used only for offline evaluation and example selection. Dataset roles are distinguished in Section 4.1.1.

## 3.2 Framework Overview

Support-bank construction and query scoring use the same frozen visual paths. Each support image is encoded, spatially mapped to the common lattice, and normalized branch by branch. All support patches are retained, with one aligned row identity across branches. The bank is fixed for a category, support seed and K. A query follows the same feature path, searches that bank, and produces patch scores under one of the two matching rules. The query never updates the bank.

The anchor combines DINOv2-B with the AnomalyCLIP visual path. Controlled variants duplicate the B descriptor or replace that copy with a different frozen encoder. Shared resizing and smoothing convert patch scores to the continuous output map, whose spatial maximum gives the image score. Both matching rules are evaluated as fixed experimental conditions. All encoders and fusion weights remain unchanged during target inference.

## 3.3 Normal Reference Matching

Let Fb denote a branch feature tensor and Rb its deterministic mapping to the common spatial lattice. At location p, branch-wise normalization produces a unit descriptor gb,p. The branch distance between a query descriptor and a support descriptor is

db(p,r) = 1 − gb,pTgb,r	(2)

All reported constructions use nonnegative distance weights wb summing to one. The descriptors in Equation (2) are nonzero; numerical norm handling follows the archived implementation. With the same aligned candidate set ℛc in every branch, joint matching is

J(p) = minr ∈ ℛcbwbdb(p,r)	(3)

Joint matching first sums the branch distances for each single candidate r, then selects the candidate with the smallest weighted distance. Equivalently, concatenating the normalized branch descriptors with coefficients equal to the square roots of their distance weights yields a unit joint descriptor. Half its squared Euclidean distance equals the weighted cosine distance. Thus, feature coefficients and distance weights must not be confused: equal thirds in distance require equal square-root coefficients, followed by the corresponding joint normalization.

Independent matching reverses the order of branch combination and candidate minimization:

L(p) = bwbminr ∈ ℛcdb(p,r)	(4)

Each branch can now select a different support patch. We denote the corresponding reference indices by rJ and rb,L when discussing selected neighbors. Ties can select different indices with identical scores, so reference identity should be interpreted together with distance rather than as a unique explanation of every query.

The difference between the two raw patch scores is

G(p) = J(p) − L(p) ≥ 0	(5)

For every candidate, each branch distance is at least its own minimum; weighting and summing preserves that inequality, and minimizing the weighted sum does not reverse it. Equality holds when the branch minima admit a common minimizer. A positive gap measures the distance cost of imposing a common reference. Localization quality must be assessed separately, because average precision depends on the ordering of normal and anomalous pixels. The same distinction applies after common linear resizing and smoothing.

## 3.4 Representation Constructions and Weight Controls

Branch labels B, S, C and D denote DINOv2-B, DINOv2-S, the AnomalyCLIP visual descriptor and WideResNet50-2, respectively. Table 1 defines the dual-branch anchor A1, duplicate control DUP, equal-weight replacement TRI and balanced replacement BAL. Branch and construction labels, matching-rule identifiers and descriptive subscripts are upright; scalar variables and score functions are italic; feature vectors and full maps are bold italic.

Table 1. Fixed representation constructions and the purpose of each control.

| Construction | Distance weights | Controlled comparison |
| --- | --- | --- |
| A1 | B 1/2; C 1/2 | Dual-visual anchor |
| DUP | B 1/3; B copy 1/3; C 1/3 | Reweight existing information; no new encoder |
| TRI | B 1/3; extra encoder 1/3; C 1/3 | Replace the duplicate at fixed slot weights |
| BAL | B 1/4; extra encoder 1/4; C 1/2 | Preserve C and total non-C weights |

Each construction is crossed with J and L. The extra slot is S or D in the main study and E1–E3 in exploratory extensions. Bold identifies the A1 anchor, not a best-performance claim.

Comparing DUP with A1 changes the effective weight of B without adding a new representation. DUP is also numerically equivalent to a two-branch B/C construction with weights two thirds and one third. Replacing the copied B descriptor in DUP with S produces TRI at identical slot weights. TRI minus DUP therefore measures the effect of that representation replacement rather than the entire difference between two and three encoders.

BAL provides a complementary comparison. In the S experiment, the combined DINO family weight remains one half, while C retains one half, allowing the B allocation to be split between B and S. In the D experiment, BAL preserves the total non-C weight but should not be called a DINO-family-preserving construction, since D is a convolutional ImageNet encoder. The weights are fixed experimental choices. DUP reuses the exact cached B descriptor, ensuring that the duplicate introduces neither new information nor feature-extraction randomness.

## 3.5 Representation Effects and Their Matching Interaction

Let P denote category-macro pixel average precision (AP), first calculated for a specified support condition and evaluation grid. Under matching rule t, the two representation effects are

ETRI,t = P(TRIt) − P(DUPt)	(6)

EBAL,t = P(BALt) − P(A1t)	(7)

The direct interaction is the difference between the same representation effect under L and J:

ITRI = ETRI,L − ETRI,J	(8)

IBAL = EBAL,L − EBAL,J	(9)

A positive interaction means that independent matching makes the representation change more beneficial or less harmful. It does not establish that the representation effect under L is itself positive. We therefore report the absolute effects together with the interactions. Conversely, a positive representation effect with an interaction interval spanning zero supports the usefulness of the representation under the observed conditions without clearly establishing dependence on matching.

The D comparison uses the same definitions with D replacing S. To compare their interactions, both are restricted to the same seeds and support budgets. For either construction label q in TRI or BAL, the encoder comparison is

ΔIq = Iq,D − Iq,S,    q ∈ {TRI, BAL}	(10)

All differences are formed within matched conditions and each resampling replicate before aggregation. This paired calculation directly estimates the interaction and the between-encoder difference, rather than comparing separate significance decisions.

## 3.6 Anomaly Outputs and Computational Cost

The patch scores form the matrix at on the common lattice, are bilinearly resized to the H by W output canvas, and are smoothed with a Gaussian of standard deviation four pixels; u indexes an output pixel. The image score is the maximum map response:

At = Gaussσ=4(ResizeH×W(at)),   simg,t = maxuAt,u	(11)

The controlled evaluation restores patch scores to the retained image canvas, with dimensions H by W; it is 448 by 448 for square images, 448 by 588 for BTAD category 03, and 630 pixels high by 224 pixels wide for KolektorSDD2. Ground truth follows the same resized and cropped image extent. Cross-method evaluation uses original-image coordinates, as described in Section 4.1.3. Pixel metrics use continuous scores. For illustrative contours only, thresholding produces

Mvis,t(u) = 1[At,u ≥ τvis]	(12)

Here τvis is a visualization rule, not a universal fixed operating threshold. The archived qualitative examples use min-max normalization followed by 256-bin Otsu thresholding. No test mask determines this threshold. Equation (12) defines a display operation; the evaluated model outputs remain the continuous anomaly map and image score. A deployment threshold would require separate calibration.

If a bank contains Nc normal patches, a query contains n patches, and the combined descriptor dimension is d, exhaustive matching requires order nNcd distance work and order Ncd feature storage. Joint matching can use one weighted concatenation index, whereas independent matching requires the corresponding branch-wise searches. Implementations can have different overheads even when the arithmetic order is the same. Larger support budgets increase reference storage, and an actual additional encoder incurs feature-extraction cost; reusing a duplicate does not incur that encoder cost. Here, training-free denotes the absence of target optimization; reference preparation and inference still incur computation.

## 3.7 Model Configuration

Table 2 records the frozen feature paths and the shared processing choices. The B and S encoders use DINOv2 patch features [6]. C uses a CLIP Vision Transformer (ViT-L/14) visual backbone with AnomalyCLIP's diagonally prominent attention map (DPAM) path [8, 23]. Its 518-pixel input yields a 37 by 37 grid; the final returned patch tensor is retained after projection, excluding the class token. Only the visual descriptor path contributes to inference.

The common lattice follows the B canvas. Inputs are resized with preserved aspect ratio to a short side of 448 pixels and cropped from the top left to dimensions divisible by 14. The C grid is mapped to the retained image extent. On BTAD category 03, the resized extent is 448 by 597 pixels and the retained canvas is 448 by 588. The corrected BTAD analysis maps both predictions and masks through the corresponding coordinates; extensions using the historical canonical mask convention are identified separately. Geometric consistency does not imply identical receptive fields or learned semantic alignment between encoders.

The D branch was specified before its results were generated. It uses ImageNet-pretrained WideResNet50-2, bilinearly resamples layer2 and layer3 to the B lattice, concatenates their 512- and 1024-channel outputs, and normalizes the resulting 1536-dimensional descriptor. Its query-feature cache is stored in float16 and converted to float32 for normalization and scoring; this storage choice is recorded in the reproduction manifest. Branch dimensions are listed separately in Table 2. Exploratory slots additionally use original DINO ViT-S/8 (E1), ConvNeXt-Tiny (E2), and Swin-Tiny (E3). They follow the same mapping and unit-normalization steps. KolektorSDD2 uses its prespecified 630 by 224 canvas for B, S and D; C retains its own input path before spatial mapping. All encoder parameters remain frozen, so target optimizer, learning rate, training epochs, and training-loss curves are not applicable.

Table 2. Frozen feature paths and shared model configuration.

| Component | Input and retained feature path | Descriptor / target training |
| --- | --- | --- |
| B | DINOv2 ViT-B/14; final patch features; short side 448 | 768 dimensions; frozen |
| S | DINOv2 ViT-S/14; final patch features; short side 448 | 384 dimensions; frozen |
| C | CLIP ViT-L/14 visual + DPAM; 518 × 518; retain layer 24 after projection | 768 dimensions; frozen |
| D | WideResNet50-2; ImageNet1K V1; layer2 + layer3 on B canvas | 512 + 1024 = 1536; frozen |
| E1 | Original DINO ViT-S/8; final patch features | 384 dimensions; frozen |
| E2 | ConvNeXt-Tiny; two retained stages | 576 dimensions; frozen |
| E3 | Swin-Tiny; two retained stages | 576 dimensions; frozen |
| Alignment | Common B grid; bilinear mapping; align_corners false | 32 × 32 for a 448 × 448 canvas |
| Normalization | Per-branch unit normalization; joint coefficients are square roots of distance weights | No learned scale or fusion parameter |
| Memory and scoring | All aligned support patches; exact nearest-neighbor matching | Fixed bank per category, seed and K |
| Output | Resize to retained H × W canvas; Gaussian standard deviation 4 pixels | Continuous map; maximum image score |
| Optimization | No target loss, optimizer, learning rate or training epochs | 0 target-trainable parameters |

C requests layers 6, 12, 18 and 24, uses a DPAM layer setting of 20 and retains the final patch tensor. No text scores enter the model. The fixed KolektorSDD2 canvas is 630 high × 224 wide; other inputs follow the B canvas. D is prespecified; E1–E3 are exploratory.

# 4 Experimental Evaluations

## 4.1 Experimental Design

### 4.1.1 Datasets and Reference Protocol

We evaluate the representation-by-matching design on the Metal Parts Defect Detection dataset (MPDD), the BeanTech Anomaly Detection dataset (BTAD), MVTec Anomaly Detection (MVTec AD), Visual Anomaly (VisA) and KolektorSDD2 [1, 2, 3, 4, 33]. Table 3 distinguishes their roles and support scopes. MPDD informed development. BTAD and MVTec AD serve as external validation datasets for the frozen comparison, although both had been explored before this analysis. VisA is in-domain validation because the inherited AnomalyCLIP checkpoint was trained on VisA. This provenance applies even though inference uses only visual descriptors. Only the KolektorSDD2 directional confirmation was specified before encoding that dataset's features.

Table 3. Dataset roles and support scopes.

| Dataset | Classes | Role | S conditions | D conditions |
| --- | --- | --- | --- | --- |
| MPDD | 6 | Development | 12: seeds 0–2 | 4: seeds 0–1; K = 1, 4 |
| BTAD | 3 | External frozen validation | 8: seeds 0–1 | 4: seeds 0–1; K = 1, 4 |
| MVTec AD | 15 | External frozen validation | 12: seeds 0–2 | Not evaluated |
| VisA | 12 | In-domain frozen validation | 12: seeds 0–2 | Not evaluated |
| KolektorSDD2 | 1 | Separate confirmation | 12: seeds 0–2 | 12: seeds 0–2; all budgets |

All S studies use K = 1, 2, 4 and 8. KolektorSDD2 uses the same four budgets for D. Supports are nested within seed and shared across paired methods. Extra-encoder comparisons on MPDD/BTAD use the four D conditions; their wider twelve-condition runs are exploratory. VisA is in-domain for the inherited C checkpoint.

For each category and seed, verified normal training images form a fixed support sequence; smaller K values are nested prefixes. The same supports and query images are used by every method in a paired comparison. Seeds 0, 1 and 2 crossed with K equal to 1, 2, 4 and 8 yield twelve dataset-level support conditions; the primary BTAD analysis uses seeds 0 and 1 and therefore eight conditions. These are repeated evaluations on common test data, not independent datasets. KolektorSDD2 supplies 1004 test images, including 110 with nonempty defect masks and 894 without defects; only verified normal training images enter the support bank.

The original D extension and the matched five-encoder comparison use seeds 0 and 1 with K equal to 1 and 4 on MPDD and BTAD. Additional E1–E3 results on twelve conditions are reported as exploratory scope checks. A separate eight-seed extension uses seeds 0 through 7 and all four budgets. Its BTAD category-03 masks follow the canonical convention, so its estimates are not substituted for the corrected-geometry primary estimates.

### 4.1.2 Controlled Constructions and External Baselines

The primary matrix includes single-branch B, S and C and both matching rules for A1, DUP, TRI and BAL. Single-branch paths diagnose descriptor quality, while the eight combined configurations identify the two interaction contrasts. The D extension adds D-only scoring and its TRI/BAL variants. E1–E3 entered after the S and D results and are exploratory encoder substitutions. They test sensitivity to the added representation.

AnomalyDINO and PatchCore provide native-method context [5, 16]. AnomalyDINO is evaluated with and without support rotation. PatchCore includes the 224-pixel input and 1024-dimensional projected-feature configuration as well as the earlier local 128-pixel and 256-dimensional configuration. Both are retained to reveal configuration sensitivity. Each method keeps its own preprocessing, features and search implementation, while support seeds and budgets are matched. This comparison assesses practical configurations and does not isolate the matching factor or constitute a comprehensive state-of-the-art benchmark.

### 4.1.3 Metrics and Statistical Inference

Pixel average precision (AP) is the primary localization metric. Scores and labels are pooled over test images within each category, then category APs are averaged equally. Dataset-level estimates average the applicable support conditions equally. This differs from pooling all categories into a single AP or averaging per-image AP. Pixel area under the receiver operating characteristic curve (AUROC) is a secondary diagnostic. Performance tables use AP on the 0–1 scale; effects and interactions use AP percentage points, equal to 100 times an AP difference.

Three spatial evaluation conventions remain separate. The main bootstrap analysis samples pixels on a regular stride-eight grid of the retained output canvas. Full-pixel estimates use every output-canvas pixel; finer stride-four checks test sensitivity to the sampled grid. External-method comparisons instead map predictions into original-image coordinates and evaluate their common valid intersection. Geometry conventions, pixel strides and support scopes are stated with each table; scores from these settings are not interchangeable.

We use 1000 paired image-level bootstrap replicates. Images are sampled with replacement within each fixed category, without additional normal/abnormal stratification. Each sampled image contributes its evaluated pixels as a block. The same deterministic replicate stream, based on seed 20260913 together with dataset identity, category identity and replicate index, is shared across methods and support conditions. Contrasts are formed within each replicate after the required category and condition aggregation. Intervals are conditional on observed categories and support manifests; they do not estimate uncertainty over unseen datasets or arbitrary future support sets.

The primary four S interactions, four D interactions and four matched D-minus-S contrasts each have their own exploratory four-comparison family, reported with 98.75% percentile intervals. The extended four-dataset S table has eight cells and reports 99.375% intervals; these are approximate Bonferroni adjustments. Encoder-transfer comparisons retain a separate four-cell family per encoder, with no simultaneous guarantee over all exploratory analyses. With 1000 replicates, extreme percentile endpoints have limited precision.

KolektorSDD2 retains its specified conjunction criterion: all four S/D interactions must be positive and their individual 95% intervals must exclude zero. This conjunction uses individual intervals and does not imply simultaneous 95% coverage. Excluding zero supports an interaction direction under the stated protocol; practical value also depends on the effect size and absolute representation gain. An interval spanning zero leaves the direction unresolved.

Observed condition-averaged point estimates and bootstrap means are labelled separately. Full-pixel estimates provide resolution checks; full-pixel bootstrap intervals are unavailable. The eight-seed study describes support sensitivity separately rather than treating its between-seed variation as part of the primary conditional interval.

### 4.1.4 Implementation and Reproducibility

The recorded machine has an NVIDIA GeForce RTX 3060 Laptop GPU with 6 GB VRAM, an Intel Core i9-12900H processor and 16 GB RAM. Encoders run in evaluation mode, with cached query features reused across controlled variants. Exact nearest-neighbor search implements Section 3, using Facebook AI Similarity Search (FAISS) where applicable [34]. Feature extraction, reference construction and score evaluation are distinct computational stages; cached-stage times cannot be interpreted as end-to-end deployment latency.

The primary BTAD tables use corrected category-03 coordinates. The four-dataset and eight-seed extensions retain their archived canonical-mask convention and are reported separately. Geometric audits verify the retained image extent and support nesting. Duplicate descriptors reuse the exact cached values. Reproduction uses support manifests, frozen branch specifications and paired bootstrap streams; no target loss is optimized and no test-mask feedback selects a normal reference.

Resource reporting is restricted to measurements whose stages and devices are identifiable. VisA's extension includes mixed CPU/GPU execution and is excluded from speed or GPU-memory rankings. Comparable complete A1 timing and PatchCore process GPU-memory measurements are unavailable.

## 4.2 Results and Analysis

### 4.2.1 Controlled Performance and the Weight Confound

Table 4 reports the eight configurations needed to identify the primary S interactions. A1 reaches pixel AP 0.3729 under J and 0.3791 under L on MPDD; corrected BTAD yields 0.6408 and 0.6498. Independent matching improves the observed anchor average, but an anchor improvement alone does not show that an additional representation benefits from the same change.

Table 4. Primary controlled matrix with the S representation.

| Configuration | MPDD AP | MPDD SD | BTAD AP | BTAD SD |
| --- | --- | --- | --- | --- |
| A1 J | 0.3729 | 0.0424 | 0.6408 | 0.0134 |
| A1 L | 0.3791 | 0.0434 | 0.6498 | 0.0129 |
| DUP J | 0.3649 | 0.0438 | 0.6316 | 0.0136 |
| DUP L | 0.3681 | 0.0445 | 0.6379 | 0.0133 |
| TRI J | 0.3626 | 0.0433 | 0.6471 | 0.0108 |
| TRI L | 0.3736 | 0.0463 | 0.6529 | 0.0086 |
| BAL J | 0.3678 | 0.0409 | 0.6505 | 0.0113 |
| BAL L | 0.3799 | 0.0437 | 0.6583 | 0.0090 |

Pixel AP on the stride-eight canvas grid. SD is the sample standard deviation across fixed support conditions: MPDD uses seeds 0, 1, 2; corrected BTAD uses seeds 0, 1; both use K = 1, 2, 4, 8. SD describes support-condition variation, not a confidence interval or independent test datasets. Bold identifies the A1 anchor.

DUP lowers AP relative to A1 under both matching rules in both datasets. On MPDD, duplicating B changes AP by approximately −0.80 percentage points under J and −1.10 under L, despite introducing no new information. The effective B weight rises from one half to two thirds while C falls to one third. This provides a concrete reason to separate branch count from representation value.

The construction ordering also differs across datasets. MPDD TRI remains below A1 under either matching rule, even though TRI L improves over its DUP control. BAL L is close to A1 L. On BTAD, TRI and BAL exceed their respective controls and BAL L has the highest observed AP within this eight-configuration matrix. These are conditional results for fixed weights; they do not identify an optimal branch set or fusion weight.

### 4.2.2 Representation Value and Matching Dependence

The absolute representation effects in Table 5 clarify what a positive interaction means. On MPDD, TRI minus DUP changes from −0.224 points under J to +0.548 under L. The L effect's 95% interval is [−0.092, +1.160], so its absolute benefit remains uncertain. BAL minus A1 similarly changes from −0.510 to +0.085 points. Removing the shared-reference constraint makes both changes more favorable without establishing that adding S improves the absolute localization performance.

BTAD separates these questions in the opposite way. TRI has positive representation effects of +1.553 and +1.503 points under J and L, respectively; both exploratory 95% intervals exclude zero. BAL also has positive effects under both rules. S is useful relative to the specified controls, but its benefit is similar under the two matching rules.

Table 5. Absolute effects of replacing the representation with S.

| Dataset | Contrast / rule | Observed effect | 95% interval |
| --- | --- | --- | --- |
| MPDD | ETRI,J | −0.224 | [−0.968, +0.478] |
|  | ETRI,L | +0.548 | [−0.092, +1.160] |
|  | EBAL,J | −0.510 | [−1.135, +0.123] |
|  | EBAL,L | +0.085 | [−0.434, +0.708] |
| BTAD | ETRI,J | +1.553 | [+0.637, +2.465] |
|  | ETRI,L | +1.503 | [+0.585, +2.495] |
|  | EBAL,J | +0.974 | [+0.177, +1.803] |
|  | EBAL,L | +0.854 | [+0.057, +1.753] |

AP percentage points on stride-eight pixels. MPDD: seeds 0, 1, 2; BTAD: seeds 0, 1 with corrected geometry. Both use K = 1, 2, 4, 8. Values are observed condition means with exploratory individual 95% intervals from 1000 paired image-bootstrap replicates.

Table 6 estimates that difference directly. MPDD has S interactions of +0.772 and +0.595 points, with 98.75% intervals [+0.346, +1.211] and [+0.189, +1.030]. Corrected BTAD has interactions of −0.050 and −0.119 points, and both intervals span zero. Thus, the primary data support positive matching dependence on MPDD while leaving its direction unresolved on BTAD.

Table 6. Direct interactions for S and the prespecified D extension.

| Dataset | Encoder | Contrast | Observed interaction | 98.75% interval |
| --- | --- | --- | --- | --- |
| MPDD | S | ITRI | +0.772 | [+0.346, +1.211] |
|  |  | IBAL | +0.595 | [+0.189, +1.030] |
|  | D | ITRI | +0.974 | [+0.514, +1.689] |
|  |  | IBAL | +0.628 | [+0.162, +1.394] |
| BTAD | S | ITRI | −0.050 | [−0.235, +0.244] |
|  |  | IBAL | −0.119 | [−0.306, +0.167] |
|  | D | ITRI | +0.622 | [+0.298, +1.003] |
|  |  | IBAL | +0.501 | [+0.188, +0.995] |

AP percentage points, stride eight. S: seeds 0, 1, 2 on MPDD and 0, 1 on corrected BTAD, with K = 1, 2, 4, 8. D: seeds 0, 1 and K = 1, 4 on both datasets. Intervals use 1000 paired image-bootstrap replicates with a separate four-comparison family per encoder. Direct between-encoder comparisons require matched conditions.

The prespecified D extension yields positive interactions on both primary datasets: +0.974 and +0.628 points on MPDD, and +0.622 and +0.501 on BTAD. All four adjusted intervals exclude zero. On the four-condition full-pixel scope, TRI D L reaches AP 0.4107 on MPDD versus 0.3554 for DUP L, and 0.6591 on BTAD versus 0.6335. Those larger differences are absolute representation gains, not the interaction itself.

A direct D-minus-S comparison restricts S to the same four conditions. The observed interaction differences on BTAD are +0.623 and +0.585 points, with adjusted intervals [+0.255, +0.945] and [+0.204, +1.060]. MPDD differences are +0.207 and +0.081 points with intervals spanning zero. This supports an encoder-specific difference on BTAD while leaving the relative interactions unresolved on MPDD. The conclusion follows from the paired between-encoder contrast.

### 4.2.3 Validation across Additional Datasets

Table 7 extends the S contrasts to MVTec AD and VisA and reports an eight-cell exploratory adjustment across the four datasets. Both interactions remain positive with intervals above zero on MPDD, MVTec AD and VisA. MVTec AD's stride-eight bootstrap means are +0.432 and +0.402 points; VisA's are +0.927 and +0.810. The full-pixel observed points retain these directions. BTAD's canonical-mask estimates remain small with intervals spanning zero.

Table 7. Extended S interactions across four datasets.

| Dataset | Contrast | Bootstrap mean | 99.375% interval | Full-pixel point |
| --- | --- | --- | --- | --- |
| MPDD | ITRI | +0.762 | [+0.324, +1.250] | +0.787 |
|  | IBAL | +0.615 | [+0.152, +1.081] | +0.601 |
| BTAD | ITRI | −0.020 | [−0.245, +0.261] | −0.045 |
|  | IBAL | −0.087 | [−0.313, +0.201] | −0.115 |
| MVTec AD | ITRI | +0.432 | [+0.309, +0.570] | +0.423 |
|  | IBAL | +0.402 | [+0.287, +0.534] | +0.386 |
| VisA | ITRI | +0.927 | [+0.738, +1.114] | +0.896 |
|  | IBAL | +0.810 | [+0.611, +1.018] | +0.760 |

AP percentage points. Bootstrap means and 99.375% intervals use stride eight, 1000 paired image-bootstrap replicates and an eight-comparison family. Full-pixel columns are observed points without intervals. Seeds 0, 1, 2 (BTAD: 0, 1); K = 1, 2, 4, 8. BTAD uses canonical masks. Differences between columns reflect both pixel sampling and the estimator summary.

The dataset roles limit the interpretation. MVTec AD adds external frozen validation, whereas VisA is in-domain for the inherited C checkpoint. The larger VisA interaction cannot be attributed causally to that provenance, but it cannot be presented as unseen-domain confirmation either. The BTAD canonical-mask row and corrected-geometry primary row differ slightly and are not substituted for each other. Both analyses leave the interaction direction uncertain.

The extended table deliberately distinguishes its bootstrap means from its full-pixel observed estimates. Differences between those columns reflect both the estimator summary and pixel sampling, so they are not a pure resolution-effect estimate. The archived paired full-pixel and stride-eight analyses provide the appropriate within-scope sensitivity check. Across the primary aggregate S and D results, their directions agree; full-pixel statistical significance is not claimed.

### 4.2.4 Separately Specified Confirmation on KolektorSDD2

KolektorSDD2 tests a directional expectation specified before feature extraction: both TRI and BAL interactions should be positive for S and D, with all four individual 95% intervals excluding zero. The internal protocol was frozen on 18 September 2026; subsequent implementation clarifications retained this decision rule.

Table 8 reports the twelve-condition result for its single category. S produces observed interactions of +0.539 and +0.343 points; D produces +0.499 and +0.367. All four individual 95% intervals exclude zero, meeting the specified conjunction criterion. The confirmation is reported separately from the eight-cell validation table and its adjustment family.

Table 8. Separately specified confirmation on KolektorSDD2.

| Encoder | Contrast | Observed interaction | 95% interval |
| --- | --- | --- | --- |
| S | ITRI | +0.539 | [+0.299, +0.823] |
|  | IBAL | +0.343 | [+0.129, +0.579] |
| D | ITRI | +0.499 | [+0.212, +0.851] |
|  | IBAL | +0.367 | [+0.128, +0.653] |

AP percentage points, stride eight, 1000 paired image-bootstrap replicates. Seeds 0, 1, 2 and K = 1, 2, 4, 8 give twelve conditions in one category. Confirmation requires all four positive interactions to have individual 95% intervals excluding zero; simultaneous 95% coverage is not asserted.

This result extends the evidence beyond the development dataset under a prospectively stated expectation. The confirmation covers one category and uses a conjunction of individual intervals. Transfer across manufacturing processes and variation in defect types and support sets require further study.

### 4.2.5 Encoder Transfer under Matched Conditions

The exploratory E1–E3 substitutions test whether the finding is specific to the two main extra encoders. Table 9 uses the same seeds and budgets for all five slots and reports observed points rather than bootstrap means. Original DINO (E1) and Swin-Tiny (E3) yield positive TRI interactions with adjusted intervals above zero on MPDD; ConvNeXt-Tiny (E2) has a small negative point and an interval spanning zero. Only E3 also has a clearly positive BAL interaction on MPDD under this scope. On BTAD, all three extra encoders have positive TRI interactions with intervals above zero; E2 and E3 also have positive BAL interactions.

Table 9. Encoder substitution under the same four support conditions.

| Encoder | MPDD TRI | MPDD BAL | BTAD TRI | BTAD BAL |
| --- | --- | --- | --- | --- |
| S | +0.767 [+0.193, +1.160] | +0.547 [+0.078, +0.962] | −0.001 [−0.189, +0.293] | −0.084 [−0.273, +0.208] |
| D | +0.974 [+0.514, +1.689] | +0.628 [+0.162, +1.394] | +0.622 [+0.298, +1.003] | +0.501 [+0.188, +0.995] |
| E1 | +0.819 [+0.246, +1.362] | +0.486 [−0.136, +1.091] | +0.568 [+0.172, +1.056] | +0.210 [−0.158, +0.649] |
| E2 | −0.019 [−0.358, +0.298] | +0.039 [−0.315, +0.588] | +0.269 [+0.152, +0.494] | +0.398 [+0.158, +0.872] |
| E3 | +1.287 [+0.644, +2.056] | +0.796 [+0.055, +1.425] | +0.478 [+0.217, +0.906] | +0.448 [+0.171, +0.955] |

Each cell gives the observed interaction in AP percentage points followed by its 98.75% interval. Stride-eight intervals use 1000 paired image-bootstrap replicates. All rows use seeds 0 and 1 with K = 1 and 4. D was prespecified; E1–E3 are post hoc. The four cells for each encoder have a separate adjustment family; no joint guarantee covers all encoder substitutions.

Paired differences against S provide a stronger comparison than ranking these points. On MPDD, the E2-minus-S TRI difference has an adjusted interval below zero, supporting a smaller interaction for this substitution. On BTAD, D, E1 and E3 exceed S for TRI, whereas D, E2 and E3 exceed S for BAL under the matched four-condition analysis. The remaining paired comparisons are inconclusive. These findings establish conditional differences for the tested encoders, not an ordering of convolutional neural networks versus transformers in general.

The wider twelve-condition E1–E3 runs change some interval decisions: E1's MPDD BAL interval excludes zero, while E3's BTAD BAL interval no longer does. These wider estimates must not be compared directly with four-condition S/D values as if only the encoder changed. Accordingly, Table 9 retains the matched scope, while wider runs serve as a sensitivity analysis. The encoder substitutions were selected after observing the main study and remain exploratory.

### 4.2.6 Support Variation and Spatial Evaluation

Table 10 summarizes the eight-seed extension. MPDD has positive point estimates under every seed for both contrasts, with mean interactions +0.679 and +0.548 points. Seven of eight individual TRI intervals and six of eight BAL intervals exclude zero at 95%. BTAD changes sign across seeds in both contrasts, and its mean points are much smaller. Counts of interval exclusions are descriptive and are not eight independent replications, because every seed uses the same test images.

Table 10. Descriptive support-set sensitivity across eight random seeds.

| Series | Mean point | SD over seeds | All points positive | 95% exclusions | Uncertainty ratio |
| --- | --- | --- | --- | --- | --- |
| MPDD ITRI | +0.679 | 0.221 | Yes | 7 of 8 | 0.48 |
| MPDD IBAL | +0.548 | 0.159 | Yes | 6 of 8 | 0.37 |
| BTAD ITRI | +0.125 | 0.136 | No | 3 of 8 | 0.68 |
| BTAD IBAL | +0.036 | 0.129 | No | 0 of 8 | 0.65 |

AP percentage points except the final ratio. Mean and sample SD summarize eight seed-level dataset-macro points after averaging categories and the four budgets within each seed; counts refer to individual 95% intervals. Seeds 0–7, K = 1, 2, 4 and 8; seeds 0–2 retain their own query features and seeds 3–7 share one cached query block. BTAD uses canonical masks. The final column uses the paired-replicate estimate of between-seed SD divided by the median test-bootstrap half-width, not simply the displayed SD divided by that width. This is a descriptive sensitivity ratio, not a variance decomposition or a confidence interval.

Support variation is not negligible beside conditional test-image uncertainty. The paired-replicate between-seed SD divided by the median test-bootstrap half-width is 0.48 for MPDD TRI and 0.68 for BTAD TRI. These ratios compare two descriptive scales, not independent variance components. The seed study also retains canonical BTAD masks, whereas the primary corrected-geometry analysis uses a different revision. The distinction prevents geometric changes from being attributed to support sampling.

Larger nested banks cannot increase an exact nearest-neighbor raw distance when descriptors remain fixed. AP and its interaction contrast do not inherit that monotonicity: they depend on the relative ordering of normal and anomalous pixels across several configurations. Per-budget and per-category analyses therefore describe observed heterogeneity rather than a monotone design law. Finer-grid BTAD analyses and corrected-coordinate checks retain the conclusion that no aggregate S interaction direction is established. Full-pixel points add a resolution check but do not replace missing full-pixel bootstrap intervals.

### 4.2.7 Correspondence Sensitivity and Its Statistical Scope

The common bank aligns branch rows by their support-image and canvas coordinates. This does not guarantee semantic correspondence of receptive fields. A separate sensitivity analysis evaluates the canvas convention, an orthogonal Procrustes transformation, a spatial permutation, and support-based optimal transport. The soft transport rule replaces a C descriptor by a weighted mixture of reference positions; an exact-assignment limit instead changes the correspondence without averaging descriptors.

Table 11. Observed interactions under correspondence transformations.

| Variant | MPDD TRI | MPDD BAL | BTAD TRI | BTAD BAL |
| --- | --- | --- | --- | --- |
| Canvas correspondence | +0.767 | +0.547 | +0.001 | −0.080 |
| Orthogonal Procrustes | +0.767 | +0.547 | +0.001 | −0.080 |
| Spatial permutation | +0.799 | +0.813 | +0.234 | +0.217 |
| Soft transport (0.1 IQR) | +0.591 | +0.570 | +0.115 | +0.103 |
| Exact assignment | +0.834 | +0.910 | +0.220 | +0.148 |
| Soft transport (0.05 IQR) | +0.580 | +0.624 | +0.162 | +0.156 |
| Soft transport (0.5 IQR) | +0.725 | +0.751 | +0.061 | +0.025 |

AP percentage points; observed means over seeds 0, 1 and K = 1, 4 in a separate correspondence-sensitivity implementation. IQR is the interquartile range of support costs. Paired image-bootstrap intervals are unavailable for these variants; the means provide descriptive sensitivity evidence and are not interchangeable with primary-pipeline estimates.

All reported MPDD mean interactions are positive, including those under soft transport and exact assignment. Table 11 averages four support conditions and reports descriptive points only. Paired image-bootstrap confidence intervals are unavailable for these variants, so their signs do not establish a population interaction direction.

The positive MPDD points show that the observed sign persists beyond the canvas convention in the tested conditions. A consistent orthogonal transformation preserves within-branch distances, making its unchanged result a numerical consistency check. Soft transport also changes the representation through descriptor averaging, so it probes both representation and correspondence sensitivity.

BTAD covers the same principal variants and transport regularization grid. Its observed means are reported alongside MPDD. Paired image-bootstrap analysis is needed to assess uncertainty in both datasets before drawing statistical robustness conclusions.

### 4.2.8 External Method Context and Resource Costs

Table 12 compares six configurations from A1 and two external method families on a common valid region. All configurations use the same four support conditions across MPDD, BTAD, MVTec AD and VisA. A1 L has the highest observed AP on MPDD, BTAD and VisA among these configurations. On MVTec AD, AnomalyDINO without rotation reaches 0.5643 and its rotation variant reaches 0.5637, both above A1 L at 0.5570. This ordering indicates dataset-dependent performance.

Rotation improves AnomalyDINO on MPDD, BTAD and VisA but slightly reduces its MVTec AD average. The 224-pixel PatchCore configuration improves over the 128-pixel configuration on every dataset. These differences show that the practical comparison is sensitive to the chosen method configuration.

The common valid region covers 76.56% of the MPDD canvas, 70.49% on average for BTAD, 76.56% for MVTec AD and 59.07% for VisA. Restricting evaluation avoids extrapolating predictions into unsupported borders but also removes those borders from the task. Backbones, augmentation and resolution still differ. Thus, the comparison supplies bounded practical context, not a full-image benchmark ranking or an isolated test of matching.

Table 12. External-method context from six configurations on a common valid region.

| Configuration | MPDD | BTAD | MVTec AD | VisA |
| --- | --- | --- | --- | --- |
| A1 L | 0.3698 | 0.6474 | 0.5570 | 0.3763 |
| A1 J | 0.3611 | 0.6380 | 0.5530 | 0.3713 |
| AnomalyDINO + rotation | 0.3214 | 0.5840 | 0.5637 | 0.3469 |
| AnomalyDINO | 0.3130 | 0.5614 | 0.5643 | 0.3291 |
| PatchCore 224 / 1024 | 0.2275 | 0.3760 | 0.4955 | 0.3111 |
| PatchCore 128 / 256 | 0.1649 | 0.2886 | 0.3966 | 0.2554 |

Category-macro pixel AP, averaged over seeds 0 and 1 and K = 1 and 4. All 864 method-category-condition rows are included. Average retained coverage is 76.56%, 70.49%, 76.56% and 59.07%, respectively. Methods differ in backbone, resolution and augmentation. Bold identifies A1, not a statistical superiority claim.

Table 13 reports native-stage timing for MPDD and BTAD where the recorded stages are interpretable. Reference rotation increases AnomalyDINO processing cost. The measurements do not include every feature-extraction stage, and PatchCore's combined processing stage cannot be separated retrospectively. A1 lacks a comparable complete historical timing record and PatchCore lacks a process GPU peak. VisA's mixed-device extension is excluded from a speed ranking.

Table 13. Recorded native-method runtime stages and process GPU memory.

| Dataset | Configuration | Processing s | Evaluation s | GPU peak MB |
| --- | --- | --- | --- | --- |
| MPDD | AnomalyDINO no rotation | 44.47 | 8.38 | 111.6–112.5 |
|  | AnomalyDINO rotation | 82.74 | 8.22 | 111.6–112.5 |
|  | PatchCore 224 | 99.75 | 23.40 | Unavailable |
| BTAD | AnomalyDINO no rotation | 53.19 | 16.39 | 118.8 |
|  | AnomalyDINO rotation | 135.38 | 16.89 | 118.8 |
|  | PatchCore 224 | 115.45 | 40.85 | Unavailable |

Mean seconds per complete dataset condition over seeds 0 and 1 and K = 1 and 4; MPDD has 458 queries and BTAD 741. Processing is bank plus retrieval for AnomalyDINO and the combined native stage for PatchCore. GPU cells report the range of process peaks across conditions; each dataset name is printed once for its group. Cached feature extraction and other unrecorded stages are not added; these are not end-to-end per-image latencies. A1 has no comparable complete historical timing.

Larger banks and genuine additional encoders require more feature storage and computation; exact duplication does not require an additional forward pass. These structural costs follow from the pipeline, while a fair latency–memory–accuracy trade-off still requires synchronized end-to-end measurements under common deployment conditions.

### 4.2.9 Interpretation and Limitations

The controlled comparisons identify three sources of a fusion gain: redistribution of existing weights, the value of a different descriptor and the dependence of that value on reference selection. MPDD and BTAD demonstrate the practical distinction. MPDD shows a positive S interaction without a clear absolute L gain, whereas BTAD shows positive absolute gains without an established interaction direction. Thus, evaluating only branch count or the final fused score can obscure the design choice responsible for the improvement.

The inference remains conditional. The primary interaction study and most dataset analyses are exploratory; the separately specified KolektorSDD2 test supplies prospective directional confirmation. VisA retains source-domain exposure through C. The bootstrap holds categories and support manifests fixed, whereas the support-seed extension describes another uncertainty source. Canonical and corrected BTAD geometry series answer closely related but distinct numerical questions. Extreme adjusted percentile endpoints are estimated from only 1000 replicates, and complete full-pixel confidence intervals are unavailable.

Encoder substitutions are exploratory. Shared normalization, alignment and smoothing support the controlled comparison; they are standard processing operations. Correspondence results provide descriptive sensitivity only; paired image-bootstrap intervals are unavailable for these variants. The observed transport behavior has no established mechanism-level explanation. External baselines cover a restricted common region and two method families rather than a comprehensive recent-method benchmark, and end-to-end efficiency remains incompletely measured.

Finally, the output is continuous anomaly evidence. A displayed thresholded contour may omit weak parts of a defect or include unrelated high-response pixels. Neither AP nor a qualitative outline establishes calibrated binary segmentation at a fixed operating point. A practical detector would require a separately specified threshold-selection protocol and evaluation of its false positives and missed defects.

# 5 Conclusion

This study separates the value of additional frozen visual representations from reweighting and normal-reference selection in few-shot industrial anomaly localization. Exact duplication and complementary fixed-weight replacements make the representation change explicit. Crossing these controls with joint and independent matching reveals whether its value depends on sharing a normal reference, beyond any absolute gain from the added encoder.

The results demonstrate why this distinction matters. DINOv2-S shows positive matching interactions on MPDD, MVTec AD and VisA, while its interaction direction remains unresolved on BTAD despite positive absolute representation effects. WideResNet50-2 yields positive interactions on both primary datasets, and the separately specified KolektorSDD2 confirmation meets all four directional criteria. Encoder and support-set analyses identify substantial variation; correspondence transformations provide additional descriptive sensitivity evidence.

The resulting design guidance is to evaluate representation composition and reference selection together under explicit weight controls. Its scope remains conditional on the tested encoders, support sets and dataset provenance. Future work should extend prospective comparisons to additional production settings, complete paired uncertainty analysis for correspondence variants, and evaluate calibrated thresholds and synchronized deployment costs. These steps would connect the controlled attribution of fusion gains to practical inspection decisions.

# Data and Code Availability

MPDD, BTAD, MVTec AD, VisA and KolektorSDD2 are distributed by their respective providers [1, 2, 3, 4, 33]. The local study archive retains support manifests, feature specifications, geometry revisions, prediction caches, per-condition metrics, bootstrap outputs and analysis scripts. A permanent public archive for the complete current study has not yet been established.

# References

[1] P. Bergmann, M. Fauser, D. Sattlegger, and C. Steger, 'MVTec AD - A comprehensive real-world dataset for unsupervised anomaly detection,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2019, pp. 9592-9600. https://doi.org/10.1109/cvpr.2019.00982

[2] S. Jezek, M. Jonak, R. Burget, P. Dvorak, and M. Skotak, 'Deep learning-based defect detection of metal parts: Evaluating current methods in complex conditions,' in Proc. 13th Int. Congr. Ultra Modern Telecommun. Control Syst. Workshops, 2021, pp. 66-71. https://doi.org/10.1109/icumt54235.2021.9631567

[3] P. Mishra, R. Verk, D. Fornasier, C. Piciarelli, and G. L. Foresti, 'VT-ADL: A vision transformer network for image anomaly detection and localization,' in Proc. IEEE 30th Int. Symp. Ind. Electron., 2021, pp. 1-6. https://doi.org/10.1109/isie45552.2021.9576231

[4] Y. Zou, J. Jeong, L. Pemula, D. Zhang, and O. Dabeer, 'SPot-the-Difference self-supervised pre-training for anomaly detection and segmentation,' in Proc. Eur. Conf. Comput. Vis. (ECCV), 2022, pp. 392-408. https://doi.org/10.1007/978-3-031-20056-4_23

[5] S. Damm, M. Laszkiewicz, J. Lederer, and A. Fischer, 'AnomalyDINO: Boosting patch-based few-shot anomaly detection with DINOv2,' in Proc. IEEE/CVF Winter Conf. Appl. Comput. Vis. (WACV), 2025, pp. 1319-1329. https://openaccess.thecvf.com/content/WACV2025/html/Damm_AnomalyDINO_Boosting_Patch-Based_Few-Shot_Anomaly_Detection_with_DINOv2_WACV_2025_paper.html

[6] M. Oquab et al., 'DINOv2: Learning robust visual features without supervision,' Trans. Mach. Learn. Res., 2024. https://openreview.net/forum?id=a68SUt6zFt

[7] X. Guo, Z. Chen, C. D. Castillo, H. Wang, and X. Liu, 'Sea-CLIP: Mining semantic-aware representations for few-shot anomaly detection with CLIP,' in Proc. IEEE/CVF Winter Conf. Appl. Comput. Vis. (WACV), 2026, pp. 3689-3699. https://openaccess.thecvf.com/content/WACV2026/html/Guo_Sea-CLIP_Mining_Semantic-Aware_Representations_for_Few-Shot_Anomaly_Detection_with_CLIP_WACV_2026_paper.html

[8] Q. Zhou, G. Pang, Y. Tian, S. He, and J. Chen, 'AnomalyCLIP: Object-agnostic prompt learning for zero-shot anomaly detection,' in Proc. Int. Conf. Learn. Represent. (ICLR), 2024. https://proceedings.iclr.cc/paper_files/paper/2024/hash/d7b50b8ac2c781a12f26155f48310d8d-Abstract-Conference.html

[9] K. Long, G. Xie, L. Ma, J. Liu, and Z. Lu, 'Revisiting Multimodal Fusion for 3D Anomaly Detection from an Architectural Perspective,' Proc. AAAI Conf. Artif. Intell., vol. 39, no. 12, pp. 12273–12281, 2025. https://ojs.aaai.org/index.php/AAAI/article/view/33337

[10] Y. Xu, H. Zhang, Y. Ma, Y. Zhu, and K. M. Ting, 'SCoNE: Spherical Consistent Neighborhoods Ensemble for Effective and Efficient Multi-View Anomaly Detection,' Proc. AAAI Conf. Artif. Intell., vol. 40, no. 19, pp. 16083–16090, 2026. https://ojs.aaai.org/index.php/AAAI/article/view/38643

[11] X.-R. Sheng, D.-C. Zhan, S. Lu, and Y. Jiang, 'Multi-View Anomaly Detection: Neighborhood in Locality Matters,' Proc. AAAI Conf. Artif. Intell., vol. 33, no. 1, pp. 4894–4901, 2019. https://ojs.aaai.org/index.php/AAAI/article/view/4418

[12] L. Cheng, Y. Wang, and X. Liu, 'Neighborhood Consensus Networks for Unsupervised Multi-view Outlier Detection,' Proc. AAAI Conf. Artif. Intell., vol. 35, no. 8, pp. 7099–7106, 2021. https://ojs.aaai.org/index.php/AAAI/article/view/16873

[13] M. Yang, J. Liu, Z. Yang, and Z. Wu, 'SLSG: Industrial image anomaly detection with improved feature embeddings and one-class classification,' Pattern Recognit., vol. 156, Art. no. 110862, 2024. https://doi.org/10.1016/j.patcog.2024.110862

[14] X. Zhang, M. Xu, and X. Zhou, 'RealNet: A feature selection network with realistic synthetic anomaly for anomaly detection,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2024, pp. 16699-16708. https://doi.org/10.1109/cvpr52733.2024.01580

[15] T. Defard, A. Setkov, A. Loesch, and R. Audigier, 'PaDiM: A patch distribution modeling framework for anomaly detection and localization,' in Pattern Recognition. ICPR International Workshops and Challenges, 2021, pp. 475-489. https://doi.org/10.1007/978-3-030-68799-1_35

[16] K. Roth, L. Pemula, J. Zepeda, B. Schölkopf, T. Brox, and P. Gehler, 'Towards total recall in industrial anomaly detection,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2022, pp. 14318-14328. https://doi.org/10.1109/cvpr52688.2022.01392

[17] Z. Hu, X. Zeng, Y. Li, Z. Yin, E. Meng, L. Zhu, and X. Kong, 'Few-shot anomaly detection with adaptive feature transformation and descriptor construction,' Chin. J. Aeronaut., vol. 38, no. 3, Art. no. 103098, 2025. https://doi.org/10.1016/j.cja.2024.06.007

[18] S. Wei, X. Wei, Z. Ma, S. Dong, S. Zhang, and Y. Gong, 'Few-shot online anomaly detection and segmentation,' Knowl.-Based Syst., vol. 300, Art. no. 112168, 2024. https://doi.org/10.1016/j.knosys.2024.112168

[19] J. Zhou, W. Wong, and F. Liao, 'One-shot unsupervised industrial anomaly detection: Enhanced performance under extreme data scarcity,' Pattern Recognit., vol. 173, Art. no. 112759, 2026. https://doi.org/10.1016/j.patcog.2025.112759

[20] Y. Li, L. Tian, Y. Dai, W. Chen, L. Bao, and X. Liu, 'FastRef: Fast prototype refinement for few-shot industrial anomaly detection,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2026, pp. 43040-43049. https://openaccess.thecvf.com/content/CVPR2026/html/Li_FastRef_Fast_Prototype_Refinement_for_Few-shot_Industrial_Anomaly_Detection_CVPR_2026_paper.html

[21] C. Lendering, E. Akdag, and E. Bondarev, 'SubspaceAD: Training-free few-shot anomaly detection via subspace modeling,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2026, pp. 28557-28566. https://openaccess.thecvf.com/content/CVPR2026/html/Lendering_SubspaceAD_Training-Free_Few-Shot_Anomaly_Detection_via_Subspace_Modeling_CVPR_2026_paper.html

[22] L. Jiang, Y. Huang, Z. Xu, Y. Xu, H.-S. Wong, and S. Wu, 'Defect cue-preserved structural feature refinement for few-shot anomaly detection,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2026, pp. 35607-35616. https://openaccess.thecvf.com/content/CVPR2026/html/Jiang_Defect_Cue-Preserved_Structural_Feature_Refinement_for_Few-Shot_Anomaly_Detection_CVPR_2026_paper.html

[23] A. Radford et al., 'Learning transferable visual models from natural language supervision,' in Proc. 38th Int. Conf. Mach. Learn., vol. 139, 2021, pp. 8748-8763. https://proceedings.mlr.press/v139/radford21a.html

[24] J. Jeong, Y. Zou, T. Kim, D. Zhang, A. Ravichandran, and O. Dabeer, 'WinCLIP: Zero-/few-shot anomaly classification and segmentation,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2023, pp. 19606-19616. https://doi.org/10.1109/cvpr52729.2023.01878

[25] X. Li, Z. Zhang, X. Tan, C. Chen, Y. Qu, Y. Xie, and L. Ma, 'PromptAD: Learning prompts with only normal samples for few-shot anomaly detection,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2024, pp. 16838-16848. https://openaccess.thecvf.com/content/CVPR2024/html/Li_PromptAD_Learning_Prompts_with_only_Normal_Samples_for_Few-Shot_Anomaly_CVPR_2024_paper.html

[26] J. Zhu and G. Pang, 'Toward generalist anomaly detection via in-context residual learning with few-shot sample prompts,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2024, pp. 17826-17836. https://doi.org/10.1109/cvpr52733.2024.01688

[27] W. Ma et al., 'AA-CLIP: Enhancing zero-shot anomaly detection via anomaly-aware CLIP,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2025, pp. 4744-4754. https://doi.org/10.1109/cvpr52734.2025.00447

[28] J. Zhu, Y.-S. Ong, C. Shen, and G. Pang, 'Fine-grained abnormality prompt learning for zero-shot anomaly detection,' in Proc. IEEE/CVF Int. Conf. Comput. Vis. (ICCV), 2025, pp. 22241-22251. https://doi.org/10.1109/iccv51701.2025.02065

[29] Z. Gu, B. Zhu, G. Zhu, Y. Chen, M. Tang, and J. Wang, 'UniVAD: A training-free unified model for few-shot visual anomaly detection,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2025, pp. 15194-15203. https://openaccess.thecvf.com/content/CVPR2025/html/Gu_UniVAD_A_Training-free_Unified_Model_for_Few-shot_Visual_Anomaly_Detection_CVPR_2025_paper.html

[30] Y. Wang, J. Peng, J. Zhang, R. Yi, Y. Wang, and C. Wang, 'Multimodal Industrial Anomaly Detection via Hybrid Fusion,' in Proc. IEEE/CVF Conf. Comput. Vis. Pattern Recognit. (CVPR), 2023, pp. 8032–8041. https://openaccess.thecvf.com/content/CVPR2023/html/Wang_Multimodal_Industrial_Anomaly_Detection_via_Hybrid_Fusion_CVPR_2023_paper.html

[31] Y. Lin et al., 'Commonality in Few: Few-Shot Multimodal Anomaly Detection via Hypergraph-Enhanced Memory,' Proc. AAAI Conf. Artif. Intell., vol. 40, no. 9, pp. 7015–7023, 2026. https://ojs.aaai.org/index.php/AAAI/article/view/37636

[32] X. Chen, X. Wang, Y. Wang, C. Han, and L. Duan, 'Learning Enhanced Representations via Contrasting for Multi-view Outlier Detection,' in Database Systems for Advanced Applications (DASFAA), 2023, pp. 110–120. https://link.springer.com/chapter/10.1007/978-3-031-30678-5_9

[33] J. Božič, D. Tabernik, and D. Skočaj, 'Mixed supervision for surface-defect detection: from weakly to fully supervised learning,' Comput. Ind., vol. 129, Art. no. 103459, 2021. https://doi.org/10.1016/j.compind.2021.103459

[34] J. Johnson, M. Douze, and H. Jegou, 'Billion-scale similarity search with GPUs,' IEEE Trans. Big Data, vol. 7, no. 3, pp. 535-547, 2021. https://doi.org/10.1109/tbdata.2019.2921572